KG3022T automatic jow device

Function:

The product has the high performance micro computer CMOS chip as core, can control the jow automatically according to the user’s time, and realize 46 times of jowing within one day, and offer the auto-control check time function for users, work accurately and operate easily, is widely used for the auto-control jow in schools, enterprises units.

Technical index:

1.       Working voltage: 220V/50Hz

2.       Applicable voltage range: 160V~240V

3.       Consumed power: <3W

4.       Time-lapse range: 0~ 59s

5.       Time error: <0.5s/day more or less

6.       Outline size: 179*139*66mm

7.       Net weight: 1100g

8.       Can set 46 ON and 46 OFF at most everyday, manual and automatically

9.       Work mode: same time everyday, different time everyday, five-day work, six-day work.

LCD instruction

Week meanings as below：

| Su | Mo | Tu | We | Th | Fr | Sa |
| --- | --- | --- | --- | --- | --- | --- |
| Sunday | Monday | Tuesday | Wednesday | Thursday | Friday | Saturday |

Other instructions:

- If you stop executive auto jow program temporally, at the clock situation, press the key “S” once, “” stops, making the jow program freezing. If you want to return the program, press the key “S” again to make “” flash, means the jow program begins working.

- IF you want jow temporally, press the key “-“(subtrahend key) at clock situation manually or press the key On/Off at the back.

- At the jow situation you want to stop jowing, press any one key;

- If the jow machine meets fault (e.g. machine dead, etc.), not work normally, you can press the key On/Off at the back manually to jow, and then eliminate faults according to the ways which are above mentioned.

- If you don’t use the product for long, please adjust the lock at the back to the close situation for saving power supply.

Operation instruction

- key instruction

The product offers five keys, they are:

Function key M: can transform the LCD contents, realize the situation of clock operation, clock setting, parameter setting, timing setting.

Select key: choose each parameter which needs adjustment of some kind of function.

Addend key +: the selected item to add upward (realize increasing by degrees)

Subtrahend key +: the selected item to reduce downward (realize reducing by degrees), can realize manual jow at clock situation;

Reposition key: Delete all the setting and return to the first situation, i.e. LCD is at the clock situation, the present time is 10:00, Sunday, jow time is 10s, and work model is same every day, without jow setting.

Please pay attention: If you don’t know how to put right or complete smoothly when you meet with faults during the cause of operation, please press function key M to the clock situation or press reposition key to the first situation to restart.

- Adjust clock

The product supply great function of adjustment, and can automatically adjust errors every day and every week according to the user’s setting.

At the clock situation, press the key M, i.e. enter the clock adjust situation, LCD is shown as picture right, and the minute position is flashing.

- At the minute flashing situation, press the keys of “+”, “-“ to the present time;

- Press the key S, the hour time position is flashing, and press the keys of “+”, “-“ to the present time;

Notes: The model of 12 Hrs and 24Hrs will appear by cycle, and the order is as below:

0→1…→11→12P→1P→…→11P→0→1→…→12→13→…→23→0

12 Hrs                                                   24Hrs

The user can choose 12 hrs model or 24hrs model according to requirements.

③ Press the key S again, the week time position begins flashing, he keys “+” to the present week (“-“ is no use at this time).

④Press the key S , enter the day clock error adjustment, as picture 2:

Pressing “+”or “-“ can adjust the day error , and the error range is -30~30s, the error mark “-“ means the clock walks too quick, need to slow everyday; the error mark “+“ means the clock walks too slow, need to quick, and the back second numbers mean the second numbers to slow or quick.

⑤ Pressing the key of “S” again, shall adjust the week error as picture 3:

④Press the key S , enter the day clock error adjustment, as picture 2:

Pressing “+”or “-“ can adjust the day error , and the error range is -30~30s, the error mark “-“ means the clock walks too quick, need to slow everyday; the error mark “+“ means the clock walks too slow, need to quick, and the back second numbers mean the second numbers to slow or quick.

⑤ Pressing the key of “S” again, shall adjust the week error as picture 3:

By pressing the keys of “+”, “-”can adjust the week error, and the error range is -15~15s, i.e. Auto slow 15s and auto quick15 every week.

⑥ Pressing the key of “M”, shall return the clock operation situation.

- system parameters setting

The user can adjust the jow time according to requirements, and can choose four working models: same every day, different every day, Five working days and Six working days.

At the clock situation, pressing the M key for 3 seconds in series, enters the system parameters setting as picture 4, and the jow time flashes at entries digit.

  - At the entries digit flashing situation, adjust the jow time by keys of “+” and “-“ and the time range is 0~59s, If the user doesn’t adjust it, the default jow time of the system is 10s.

  - Press the key of S, the LCD shall show as picture 5, and enter the working models to choose by pressing “+” or “-“.

Four working models：

First model: same everyday, LCD shows “Su Mo Tu We Th Fr Sa”；

Second model: different every day, LCD shows MO, cycle after seven days;

Third model: Five working days: same from Monday to Friday, LCD shows “Mo Tu We Th Fr ”,

Fourth model: six working days: same from Monday to Saturday, LCD shows “Mo Tu We Fr Sa”；if user don’t choose, the default model is same everyday.

  - Pressing M key again, return the clock operation situation.

- Time setting

At time setting function, the user can put in the jow time requirement in order, according to the user’s requirement, after opening, the jow device shall jow at the specified time according to user’s setting.

Before setting, the user must pay attention: ① First list the jow time in order②confirm the jow working model; ③confirm the week no. of every time jow, and put into the jow device.

At clock operation situation, keep pressing M key twice, enter the situation, and LCD shows as picture 6, and the entries digit program group flashing.

- At the program group flashing situation (i.e. jow time order no.), press the keys of “+” and “-“ to enter the jow program group( first enter, the system default is 01 time, no need setting)

- Press the s key to make the position of “=:=” jump, and press the keys of “+” and “-“ to adjust the min no. of jow.

- Press the s key again to make the hour digit jump, and press the keys of “+” and “-“ to adjust the min no. of jow. And then confirm the jow time.

- Press the s key again to make the week digit jump, and press the keys of “+ to confirm which day to work, and the week choice differ from working model.

First model: same everyday, no need setting week;

Second model: different every day, offers choice from Monday to Sunday.

Third model: Five working days, offers four choices form Monday to Friday, Saturday, Sunday, Saturday and Sunday.

Fourth model: six working days, offers two choices form Monday to Saturday, Sunday.

- Repeat steps of ①~④, and set other jow time according to jow program in order.

- After setting, Press the s key to make program no. jump, and press the keys of “+” and “-“ to check whether the jow time is correct or not. The jow program shall be in time order, and adjust if it has confusion.

- After checking, you must press M key, then return the clock situation. Otherwise, the jow device shall be at waiting situation.

Thus, all the settings shall be finished.

Fault elimination

If the jow device doesn’t work as you set, may be caused by the following reasons:

- The key S may be artificial done, and the jow program is frozen, please check the“” mark of frozen/ activation at the up right corner of LCD whether it is flashing or not, if static, please press the key S to make flash, thus return the jow activation situation.

- The work model of parameter setting you chosen is not correct, please readjust as introduced.

- The week adjustment is wrong at the time setting, please readjust as introduced.

- The jow time is not according to the actual time order, please readjust as introduced.

- You set unwanted program group at the time setting, please delete ( Notice: if the program group time shows “0: = = “, means no jow setting, don’t mistaken as “00:00” means cancel;

- If the program set is normal, while the switch doesn’t work normally, please check the fuse whether it is melt. If yes, please change a new one with 0.1A~0.3A( 0.5A is for AC/ DC use)

- If the jow device is dead, i.e. pressing four keys still not work, you can press the return key to readjust all the setting.

- Addition, if the jow device is at the wet/ corrosive/ with high metal gas places, or the product is with oil or water, which also affects the jow device to work normally.

- You have not yet eliminate the faults if you use the above mentioned ways , please contact our company or the local dealer.

Connection instruction

According to the clue on the shell, connect the 220V AC to the “ AC current input” terminal, and connect 220V electric bell (power less than 3000W) to “AC current output” terminal. If AC/DC use, you shall connect DC electric bell to the DC input terminal, and connect the storage battery (12V/4Ah~7Ah) to the DC input terminal, and can realize the auto transformation to jow after power cut.

When connection, please notice:

- the storage battery is less than 12V/7Ah;

- Corresponding the “+” and “-“ of storage battery to the shell marked, otherwise will burn out the jow device;

③You’d better not connect DC electric bell when it is not connected with storage battery.